Communication Protocol

Communication Protocol

V2.1

Invelion Co., Ltd. All Right Reserved

| Version | Day | Description |
| --- | --- | --- |
| V1.1 | 2013/10/15 | Initial version. |
| V1.2 | 2013/12/19 | Add the IO control command |
| V2.0 | 2014/5/20 | Modify Read, Write, Lock, Kill commands by removing PC and EPC data field |
| V2.1 | 2015/04/28 | Add NXP and Impinj Monza tags’ commands. |

1.	Contents

1.	Contents............................................................................................................................................................ 3

2.	Internal MCU..................................................................................................................................................... 5

3.	Command frame............................................................................................................................................... 7

3.1.	Command Frame structure........................................................................................................................7

3.2.	Command Type.......................................................................................................................................... 7

4.	Command Definition......................................................................................................................................... 8

4.1 Get Module Information................................................................................................................................. 8

4.2.	Single Inventory......................................................................................................................................... 9

4.3.	Multiple Inventory................................................................................................................................... 11

4.4.	Stop Multiple Inventory........................................................................................................................... 12

4.5.	Set Select Parameters.............................................................................................................................. 13

4.6.	Set Select Mode....................................................................................................................................... 14

4.7.	Read......................................................................................................................................................... 15

4.8.	Write........................................................................................................................................................ 17

4.9.	Lock.......................................................................................................................................................... 19

4.10.	Kill.........................................................................................................................................................22

4.11.	Get Query Parameters......................................................................................................................... 24

4.12.	Set Query Parameters.......................................................................................................................... 25

4.13.	Set Region............................................................................................................................................ 26

4.14.	Get Region............................................................................................................................................27

4.15.	Set Channel.......................................................................................................................................... 28

4.16.	Get Channel..........................................................................................................................................29

4.17.	Frequency Hopping.............................................................................................................................. 30

4.18.	Get TX Power........................................................................................................................................31

4.19.	Set TX Power........................................................................................................................................ 32

4.20.	Continuous Wave................................................................................................................................. 33

4.21.	Get Modem Parameters...................................................................................................................... 34

4.22.	Set Modem Parameters....................................................................................................................... 36

4.23.	Scan Jammer........................................................................................................................................ 38

4.24.	Scan Channel (LBT)...............................................................................................................................39

4.25.	GPIO..................................................................................................................................................... 40

4.26.	Module Sleep....................................................................................................................................... 42

4.27.	Set Module Idle Time........................................................................................................................... 43

4.28.	NXP ReadProtect/Reset ReadProtect.................................................................................................. 44

4.29.	NXP Change EAS................................................................................................................................... 46

4.30.	NXP EAS_Alarm.................................................................................................................................... 48

4.31.	NXP ChangeConfig............................................................................................................................... 49

4.32.	Impinj Monza QT..................................................................................................................................51

5.	Summary......................................................................................................................................................... 53

6.	Error Code....................................................................................................................................................... 54

2.	Internal MCU

M100 has an internal 8bit 8051 MCU，which has 256Byte RAM, 16Kbyte program ROM and 3 timer (In the firmware, the timer2 is used for baud rate control, the timer0 is used for frequency hopping control and the timer1 is reserved for user). And there is 8Kbyte data RAM which can be used by the 8051 MCU and the MODEM. In the receiving phase, the data RAM can NOT be accessed by MCU.

There is no internal flash in M100. The firmware of MCU could be download by UART or I2C (GPIO

emulated: P1.0, P1.1) interface.

UART interface frame has 8bit data and 1bit stop, no parity.

The handshake protocol for firmware download at power up or reset is as below:

Baud Rate Settings

| Parameter | Baud Rate(bps) |
| --- | --- |
| 0xB0 | 9600 |
| 0xB1 | 19200 |
| 0xB2 | 28800 |
| 0xB3 | 38400 |
| 0xB4 | 57600 |
| 0xB5 | 115200 |

3.	Command frame

3.1.Command Frame structure

Every command includs: Header, Command Type, Command Code, Parameter Length (PL), Checksum, End.

Example:

| Header | Type | Command | PL(MSB) | PL(LSB) | Parameter | Checksum | End |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BB | 00 | 07 | 00 | 01 | 01 | 09 | 7E |

Header:	0xBB Type:	0x00

Command:	0x07

PL:	0x0001

Parameter:	0x01

Checksum:	0x09

End:	0x7E

Checksum is calculated by: Sum from Type to Parameter. And take the least significant Byte of the sum.

3.2.Command Type

| Type | Description |
| --- | --- |
| 0x00 | Command frame: command received by firmware |
| 0x01 | Response frame: firmware response to the received command |
| 0x02 | Notice frame: firmware initiated report |

Every received command has an corresponding response which returns the command execution result.

Notice frame is used for single or multiple inventory. Firmware sends notice frame every time when read tag successfully.

4.	Command Definition

4.1 Get Module Information

4.1.1.   Command frame

This command can get the information of modules such as hardware version, software version and manufacturer information.

Frame type：	0x00

Command code：	0x03

Parameter：

Hardware version：	0x00

Software version：	0x01

Manufacturer:	0x02

For example, get the hardware version:

| Header | Type | Command | PL(MSB) | PL(LSB) | Parameter | Checksum | End |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BB | 00 | 03 | 00 | 01 | 00 | 04 | 7E |

Type:	0x00

Command:	0x03

PL:	0x0001

Parameter:	0x00(hardware version) Checksum:	0x04

4.1.2.   Response frame

Frame type：	0x01

Command code：	0x03

Data：	variables (ASCII codes)

The response data first byte is the information type, followed by the detailed message in ASCII codes. The response of getting hardware version is as blow：

| Header | Type | Command | PL(MSB) | PL(LSB) | Info Type | Info |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BB | 01 | 03 | 00 | 10 | 00 | 4D (‘M’) | 31 (‘1’) |
| 30 (‘0’) | 30 (‘0’) | 20 (‘ ‘) | 32 (‘2’) | 36 (‘6’) | 64 (‘d’) | 42 (‘B’) | 6D (‘m’) |
|  |  |  |  |  | Checksum | End |  |
| 20 (‘ ‘) | 56 (‘V’) | 31(‘1’) | 2E (‘.’) | 30 (‘0’) | 22 | 7E |  |

Type:	0x01

Command:	0x03

PL:	0x0010

Info Type:	0x00

Info:	4D 31 30 30 20 32 36 64 42 6D 20 56 31 2E 30 (“M100 26dBm V1.0” in ASCII codes) Checksum:	0x22

4.2. Single Inventory

4.2.1.   Command frame

When received this command, the reader chip will do the inventory once. The power amplifier will be opened at the beginning of the inventory and be closed after the inventory. Select parameter settings please refer to section 4.4. Query parameter settings please refer to section 4.11.

Single inventory command is as below:

| Header | Type | Command | PL(MSB) | PL(LSB) | Checksum | End |
| --- | --- | --- | --- | --- | --- | --- |
| BB | 00 | 22 | 00 | 00 | 22 | 7E |

Type:	0x00

Command:	0x22

PL:	0x0000

Checksum:	0x22

4.2.2.   Notice frame

After the firmware received a single inventory command, and successfully read a tag (CRC check passed)， a notice frame including RSSI, PC, EPC and CRC will be sent. If multiple tags are read in one inventory, notice frames of the same number of successfully read tag will be sent.

The notice frame is as below:

| Header | Type | Command | PL(MSB) | PL(LSB) | RSSI | PC(MSB) | PC(LSB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BB | 02 | 22 | 00 | 11 | C9 | 34 | 00 |
| EPC(MSB) |  |  |  |  |  |  |  |
| 30 | 75 | 1F | EB | 70 | 5C | 59 | 04 |
|  |  |  | EPC(LSB) | CRC(MSB) | CRC(LSB) | Checksum | End |
| E3 | D5 | 0D | 70 | 3A | 76 | EF | 7E |

Type:	0x02

Command:	0x22

PL:	0x0011

RSSI:	0xC9

PC:	0x3400

EPC:	0x30751FEB705C5904E3D50D70

CRC:	0x3A76

Checksum:	0xEF

RSSI is a received signal strength indicator at the RF input port of the reader chip. It's a complementally coded signed hex in the unit of dBm. E.g. an RSSI of 0xC9 means the tag returned signal power at the input

port of the reader chip is -55dBm。

4.2.3.   Response frame

If there is no tag response, or a CRC error occurred, an error code of 0x15 will be sent in response frame：

| Header | Type | Command | PL(MSB) | PL(LSB) | Parameter | Checksum | End |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BB | 01 | FF | 00 | 01 | 15 | 16 | 7E |

Type:	0x01

Command:	0xFF PL:	0x01

Parameter:	0x15

Checksum:	0x16

4.3. Multiple Inventory

4.3.1.   Command frame

When received this command, the reader chip will do the inventory by multiple times. The value of inventory time could be between 0 to 65535.

The following is an example for 10000 times inventory command:

| Header | Type | Command | PL(MSB) | PL(LSB) | Reserved | CNT(MSB) | CNT(LSB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BB | 00 | 27 | 00 | 03 | 22 | 27 | 10 |
| Checksum | End |  |  |  |  |  |  |
| 83 | 7E |  |  |  |  |  |  |

Type:	0x00

Command:	0x27

PL:	0x0003

Reserved:	0x22

CNT:	0x2710

Checksum:	0x83

4.3.2.   Notice frame

The notice frame of multiple inventory is the same as single inventory:

| Header | Type | Command | PL(MSB) | PL(LSB) | RSSI | PC(MSB) | PC(LSB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BB | 02 | 22 | 00 | 11 | C9 | 34 | 00 |
| EPC(MSB) |  |  |  |  |  |  |  |
| 30 | 75 | 1F | EB | 70 | 5C | 59 | 04 |
|  |  |  | EPC(LSB) | CRC(MSB) | CRC(LSB) | Checksum | End |
| E3 | D5 | 0D | 70 | 3A | 76 | EF | 7E |

Type:	0x02

Command:	0x22

PL:	0x0011

RSSI:	0xC9

PC:	0x3400

EPC:	0x30751FEB705C5904E3D50D70

CRC:	0x3A76

Checksum:	0xEF

4.3.3.   Response frame

If there is no tag response, or a CRC error occurred, an error code of 0x15 will be sent in response frame:

| Header | Type | Command | PL(MSB) | PL(LSB) | Parameter | Checksum | End |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BB | 01 | FF | 00 | 01 | 15 | 16 | 7E |

Type:	0x01

Command:	0xFF PL:	0x01

Parameter:	0x15

Checksum:	0x16

4.4. Stop Multiple Inventory

4.4.1.   Command frame

The multiple inventory could be stopped (not pause) immediately by this command:

| Header | Type | Command | PL(MSB) | PL(LSB) | Checksum | End |
| --- | --- | --- | --- | --- | --- | --- |
| BB | 00 | 28 | 00 | 00 | 28 | 7E |

Type:	0x00

Command:	0x28

PL:	0x0000

Checksum:	0x28

4.4.2.   Response frame

If the multiple inventory is stopped successfully, a response frame will be return:

| Header | Type | Command | PL(MSB) | PL(LSB) | Parameter | Checksum | End |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BB | 01 | 28 | 00 | 01 | 00 | 2A | 7E |

Type:	0x01

Command:	0x28

PL:	0x0001

Parameter:	0x00

Checksum:	0x2A

4.5. Set Select Parameters

4.5.1.   Command frame

This command will set select parameters and set Select Mode to 0x02. And the select will be send before inventory by default. Example (select tags which has the same code from bit 0x20 to bit 0x80):

| Header | Type | Command | PL(MSB) | PL(LSB) | SelParam | Ptr(MSB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BB | 00 | 0C | 00 | 13 | 01 | 00 | 00 |
|  | Ptr(LSB) | MaskLen | Truncate | Mask(MSB) |  |  |  |
| 00 | 20 | 60 | 00 | 30 | 75 | 1F | EB |
|  |  |  |  |  |  |  | Mask(LSB) |
| 70 | 5C | 59 | 04 | E3 | D5 | 0D | 70 |
| Checksum | End |  |  |  |  |  |  |
| AD | 7E |  |  |  |  |  |  |

Type:	0x00

Command:	0x0C PL	0x0013

SelParam:	0x01 (Target: 3’b000, Action: 3’b000, MemBank: 2’b01) Ptr:	0x00000020(in bit) from EPC first bit

MaskLen:	0x60

Truncate:	0x00(0x00: Disable truncation，0x80: Enable truncation) Mask:	0x30751FEB705C5904E3D50D70

Checksum:	0xAD

SelParam is a one byte parameter which contains 3-bits Target, 3-bits Action and 2-bits MemBank. The meaning of MemBank is:

2’b00:	RFU Memory Bank

2’b01:	EPC Memory Bank

2’b02:	TID Memory Bank

2’b03:	User Memory Bank

The meanings of Target and Action please refer to EPC UHF Class 1 Gen 2 Standard Protocol.

4.5.2.   Response frame

When select parameters are set successfully, a response frame will be return:

| Header | Type | Command | PL(MSB) | PL(LSB) | Data | Checksum | End |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BB | 01 | 0C | 00 | 01 | 00 | 0E | 7E |

Type:	0x01

Command:	0x0C PL:	0x0001

Data:	0x00

Checksum:	0x0E

4.6. Set Select Mode

4.6.1.   Command frame

After set select parameters, the select command could be set to send or not send before inventory or other operation. Example (not to send select before inventory):

| Header | Type | Command | PL(MSB) | PL(LSB) | Mode | Checksum | End |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BB | 00 | 12 | 00 | 01 | 01 | 14 | 7E |

Type:	0x00

Command:	0x12

PL:	0x0001

Mode:	0x01

Checksum:	0x14

The meaning of select Mode：

0x00:	Send select before all tag operation to select a certain tag.

0x01:	Not to send select before all tag operation.

0x02:	Send select before all tag operation except Inventory (Single Inventory and Multiple

Inventory).E.g. select a certain tag before Read, Write, Lock, and Kill.

4.6.2.   Response frame

When the select is set to be send or not send successfully, a response frame will be return:

| Header | Type | Command | PL(MSB) | PL(LSB) | Data | Checksum | End |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BB | 01 | 0C | 00 | 01 | 00 | 0E | 7E |

Type:	0x01

Command:	0x0C PL:	0x0001

Data:	0x00(success) Checksum:	0x0E

4.7. Read

4.7.1.   Command frame

Read the memory bank of tag from the given address and by the given length. The unit for segment address (SA) and data length (DL) is word. Before this command, a set select parameter command should be sent first to select a signal tag. The access command will not be send if the access password is all zero.

| Header | Type | Command | PL(MSB) | PL(LSB) | AP(MSB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BB | 00 | 39 | 00 | 09 | 00 | 00 | FF |
| AP(LSB) | MemBank | SA(MSB) | SA(LSB) | DL(MSB) | DL(LSB) | Checksum | End |
| FF | 03 | 00 | 00 | 00 | 02 | 45 | 7E |

Type:	0x00

Command:	0x39

PL	0x0009

Access Password:	0x0000FFFF

MemBank:	0x03(User Memory Bank) Segment Address:	0x0000

Data Length:	0x0002

Checksum:	0x45

4.7.2.   Response frame

If the required data is read successfully, a response frame will be returned:

| Header | Type | Command | PL(MSB) | PL(LSB) | UL | PC(MSB) | PC(LSB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BB | 01 | 39 | 00 | 13 | 0E | 34 | 00 |
| EPC(MSB) |  |  |  |  |  |  |  |
| 30 | 75 | 1F | EB | 70 | 5C | 59 | 04 |
|  |  |  | EPC(LSB) | Data(MSB) |  |  | Data(LSB) |
| E3 | D5 | 0D | 70 | 12 | 34 | 56 | 78 |
| Checksum | End |  |  |  |  |  |  |
| B0 | 7E |  |  |  |  |  |  |

Type:	0x01

Command:	0x39

PL:	0x0013

PC+EPC length:	0x0E PC:	0x3400

EPC:	0x30751FEB705C5904E3D50D70

Data:	0x12345678

Checksum:	0x52

If there is no tag response, an error code of 0x09 will be returned:

| Header | Type | Command | PL(MSB) | PL(LSB) | Error Code | Checksum | End |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BB | 01 | FF | 00 | 01 | 09 | 0A | 7E |

Type:	0x01

Command:	0xFF PL:	0x0001

Error Code:	0x09

Checksum:	0x0A

If the access password is not correct, an error code of 0x16 will be returned, and the tag’s PC+EPC will be sent back:

| Header | Type | Command | PL(MSB) | PL(LSB) | Error Code | UL | PC(MSB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BB | 01 | FF | 00 | 10 | 16 | 0E | 34 |
| PC(LSB) | EPC(MSB) |  |  |  |  |  |  |
| 00 | 30 | 75 | 1F | EB | 70 | 5C | 59 |
|  |  |  |  | EPC(LSB) | Checksum | End |  |
| 04 | E3 | D5 | 0D | 70 | 75 | 7E |  |

Type:	0x01

Command:	0xFF PL:	0x0010

Error Code:	0x16

PC+EPC length:  0x0E PC:	0x3400

EPC:	0x30751FEB705C5904E3D50D70

Checksum:	0x75

If there are errors in SA or DL (e.g. the reading address is exceed the data memory bound), an error code of

0xA3 will be returned, and the tag’s PC+EPC will be sent back:

| Header | Type | Command | PL(MSB) | PL(LSB) | Error Code | UL | PC(MSB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BB | 01 | FF | 00 | 10 | A3 | 0E | 34 |
| PC(LSB) | EPC(MSB) |  |  |  |  |  |  |
| 00 | 30 | 75 | 1F | EB | 70 | 5C | 59 |
|  |  |  |  | EPC(LSB) | Checksum | End |  |
| 04 | E3 | D5 | 0D | 70 | 02 | 7E |  |

Type:	0x01

Command:	0xFF PL:	0x0010

Error Code:	0xA3

PC+EPC length:  0x0E PC:	0x3400

EPC:	0x30751FEB705C5904E3D50D70

Checksum:	0x02

4.8. Write

4.8.1.   Command frame

Write the memory bank of tag from the given address and by the given length. The unit for segment address (SA) and data length (DL) is word. Before this command, a set select parameter command should be sent first to select a signal tag. The access command will not be send if the access password is all zero. The data length to be written cannot exceed 32 words (64 bytes).

| Header | Type | Command | PL(MSB) | PL(LSB) | AP(MSB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BB | 00 | 49 | 00 | 0D | 00 | 00 | FF |
| AP(LSB) | MemBank | SA(MSB) | SA(LSB) | DL(MSB) | DL(LSB) | DT(MSB) |  |
| FF | 03 | 00 | 00 | 00 | 02 | 12 | 34 |
|  | DT(LSB) | Checksum | End |  |  |  |  |
| 56 | 78 | 6D | 7E |  |  |  |  |

Type:	0x00

Command:	0x39

PL:	0x000D Access Password:	0x0000FFFF MemBank:	0x03

Segment Address:	0x0000

Data Length:	0x0004

Data:	0x12345678

Checksum:	0x6D

4.8.2.   Responds frame

If the data is written successfully, a response frame will be returned:

| Header | Type | Command | PL(MSB) | PL(LSB) | UL | PC(MSB) | PC(LSB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BB | 01 | 49 | 00 | 10 | 0E | 34 | 00 |
| EPC(MSB) |  |  |  |  |  |  |  |
| 30 | 75 | 1F | EB | 70 | 5C | 59 | 04 |
|  |  |  | EPC(LSB) | Parameter | Checksum | End |  |
| E3 | D5 | 0D | 70 | 00 | A9 | 7E |  |

Type:	0x01

Command:	0x49

PL:	0x0010

PC+EPC length:	0x0E PC:	0x3400

EPC:	0x30751FEB705C5904E3D50D70

Parameter:	0x00 (execute success) Checksum:	0xA9

If there is no tag response, an error code of 0x10 will be returned:

| Header | Type | Command | PL(MSB) | PL(LSB) | Error Code | Checksum | End |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BB | 01 | FF | 00 | 01 | 10 | 0A | 7E |

Command:	0xFF PL:	0x0001

Error Code:	0x10

Checksum:	0x0A

If the access password is not correct, an error code of 0x16 will be returned, and the tag’s PC+EPC will be sent back:

| Header | Type | Command | PL(MSB) | PL(LSB) | Error Code | UL | PC(MSB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BB | 01 | FF | 00 | 10 | 16 | 0E | 34 |
| PC(LSB) | EPC(MSB) |  |  |  |  |  |  |
| 00 | 30 | 75 | 1F | EB | 70 | 5C | 59 |
|  |  |  |  | EPC(LSB) | Checksum | End |  |
| 04 | E3 | D5 | 0D | 70 | 75 | 7E |  |

Type:	0x01

Command:	0xFF PL:	0x0010

Error Code:	0x16

PC+EPC length:	0x0E PC:	0x3400

EPC:	0x30751FEB705C5904E3D50D70

Checksum:	0x75

If there is an error code according to EPC Gen2 Protocol occurs (e.g. the address exceeds the data memory bound, an error code of 0xB3 will be returned), the error code bitwise OR 0xB0 will be returned, and the tag’s PC+EPC will be sent back:

| Header | Type | Command | PL(MSB) | PL(LSB) | Error Code | UL | PC(MSB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BB | 01 | FF | 00 | 10 | B3 | 0E | 34 |
| PC(LSB) | EPC(MSB) |  |  |  |  |  |  |
| 00 | 30 | 75 | 1F | EB | 70 | 5C | 59 |
|  |  |  |  | EPC(LSB) | Checksum | End |  |
| 04 | E3 | D5 | 0D | 70 | 12 | 7E |  |

Type:	0x01

Command:	0xFF PL:	0x0010

Error Code:	0xB3

PC+EPC length:	0x0E PC:	0x3400

EPC:	0x30751FEB705C5904E3D50D70

Checksum:	0x12

The meaning of error code:

0xB0:	Other Error

0xB3:	Memory Overrun

0xB4:	Memory Locked

0xBB:	Insufficient Power

4.9. Lock

4.9.1.   Command frame

Lock or unlock the tag memory band by access password. Before this command, a set select parameter command should be sent first to select a signal tag.

| Header | Type | Command | PL(MSB) | PL(LSB) | AP(MSB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BB | 00 | 82 | 00 | 07 | 00 | 00 | FF |
| AP(LSB) | LD(MSB) |  | LD(LSB) | Checksum | End |  |  |
| FF | 02 | 00 | 80 | 09 | 7E |  |  |

Type:	0x00

Command:	0x82

PL:	0x0007

Access Password:	0x0000FFFF Lock payload:	0x020080

Checksum:	0x09

The 4bits MSB of the lock payload (LD) is reserved. And the next 20 bits is for lock mask (10 bits) and lock action (10 bits) accordingly.

The detailed lock payload, masks and action field should refer to EPCTM Radio-Frequency Identity Protocols from EPC global

4.9.2.   Response frame

If lock operation is successes, a response frame will be returned:

| Header | Type | Command | PL(MSB) | PL(LSB) | UL | PC(MSB) | PC(LSB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BB | 01 | 82 | 00 | 10 | 0E | 34 | 00 |
| EPC(MSB) |  |  |  |  |  |  |  |
| 30 | 75 | 1F | EB | 70 | 5C | 59 | 04 |
|  |  |  | EPC(LSB) | Parameter | Checksum | End |  |
| E3 | D5 | 0D | 70 | 00 | E2 | 7E |  |

Type:	0x01

Command:	0x82

PL:	0x0010

PC+EPC length:  0x0E PC:	0x3400

EPC:	0x30751FEB705C5904E3D50D70

Parameter:	0x00

Checksum:	0xE2

If there is no tag response, an error code of 0x13 will be returned:

| Header | Type | Command | PL(MSB) | PL(LSB) | Error Code | Checksum | End |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BB | 01 | FF | 00 | 01 | 13 | 14 | 7E |

Type:	0x01

Command:	0xFF PL:	0x0001

Error Code:	0x13

Checksum:	0x14

If the access password is not correct, an error code of 0x16 will be returned, and the tag’s PC+EPC will be sent back:

| Header | Type | Command | PL(MSB) | PL(LSB) | Error Code | UL | PC(MSB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BB | 01 | FF | 00 | 10 | 16 | 0E | 34 |
| PC(LSB) | EPC(MSB) |  |  |  |  |  |  |
| 00 | 30 | 75 | 1F | EB | 70 | 5C | 59 |
|  |  |  |  | EPC(LSB) | Checksum | End |  |
| 04 | E3 | D5 | 0D | 70 | 75 | 7E |  |

Type:	0x01

Command:	0xFF PL:	0x0010

Error Code:	0x16

PC+EPC length:	0x0E PC:	0x3400

EPC:	0x30751FEB705C5904E3D50D70

Checksum:	0x75

If there is an error code according to EPC Gen2 Protocol occurs (e.g. the Memory Bank has been permalocked, an error code of 0xC4 will be returned), the error code bitwise OR 0xC0 will be returned, and the tag’s PC+EPC will be sent back:

| Header | Type | Command | PL(MSB) | PL(LSB) | Error Code | UL | PC(MSB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BB | 01 | FF | 00 | 10 | C4 | 0E | 34 |
| PC(LSB) | EPC(MSB) |  |  |  |  |  |  |
| 00 | 30 | 75 | 1F | EB | 70 | 5C | 59 |
|  |  |  |  | EPC(LSB) | Checksum | End |  |
| 04 | E3 | D5 | 0D | 70 | 23 | 7E |  |

Type:	0x01

Command:	0xFF PL:	0x0010

Error Code:	0xC4

PC+EPC length:	0x0E PC:	0x3400

EPC:	0x30751FEB705C5904E3D50D70

Checksum:	0x23

The meaning of error code:

0xC0:	Other Error

0xC3:	Memory Overrun

0xC4:	Memory Locked

0xCB:	Insufficient Power

4.10. Kill

4.10.1. Command frame

Before this command, a set select parameter command should be sent first to select a signal tag. The kill command is as follow:

| Header | Type | Command | PL(MSB) | PL(LSB) | KP(MSB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BB | 00 | 65 | 00 | 04 | 00 | 00 | FF |
| KP(LSB) | Checksum | End |  |  |  |  |  |
| FF | 67 | 7E |  |  |  |  |  |

Type:	0x00

Command:	0x65

PL:	0x0004

Kill Password:	0x0000FFFF Checksum:	0x67

Notice: A tag with all zero Kill Password cannot be killed!

4.10.2. Response frame

If kill operation is successes and the tag responded CRC is correct, a response frame will be returned:

| Header | Type | Command | PL(MSB) | PL(LSB) | UL | PC(MSB) | PC(LSB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BB | 01 | 65 | 00 | 10 | 0E | 34 | 00 |
| EPC(MSB) |  |  |  |  |  |  |  |
| 30 | 75 | 1F | EB | 70 | 5C | 59 | 04 |
|  |  |  | EPC(LSB) | Parameter | Checksum | End |  |
| E3 | D5 | 0D | 70 | 00 | C5 | 7E |  |

Type:	0x01

Command:	0x65

PL:	0x0010

PC+EPC length:  0x0E PC:	0x3400

EPC:	0x30751FEB705C5904E3D50D70

Parameter:	0x00

Checksum:	0xC5

If there is no tag response, an error code of 0x12 will be returned:

| Header | Type | Command | PL(MSB) | PL(LSB) | Error Code | Checksum | End |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BB | 01 | FF | 00 | 01 | 12 | 13 | 7E |

Type:	0x01

Command:	0xFF PL:	0x0001

Error Code:	0x12

Checksum:	0x13

If there is an error code according to EPC Gen2 Protocol occurs (e.g. the tag has no kill password, an error code of 0xD0 will be returned), the error code bitwise OR 0xD0 will be returned, and the tag’s PC+EPC will

be sent back:

| Header | Type | Command | PL(MSB) | PL(LSB) | Error Code | UL | PC(MSB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BB | 01 | FF | 00 | 10 | D0 | 0E | 34 |
| PC(LSB) | EPC(MSB) |  |  |  |  |  |  |
| 00 | 30 | 75 | 1F | EB | 70 | 5C | 59 |
|  |  |  |  | EPC(LSB) | Checksum | End |  |
| 04 | E3 | D5 | 0D | 70 | 2F | 7E |  |

Type:	0x01

Command:	0xFF PL:	0x0001

Error Code:	0xD0

PC+EPC length:  0x0E PC:	0x3400

EPC:	0x30751FEB705C5904E3D50D70

Checksum:	0x2F

4.11. Get Query Parameters

4.11.1. Command frame

To get query parameters:

| Header | Type | Command | PL(MSB) | PL(LSB) | Checksum | End |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BB | 00 | 0D | 00 | 00 | 0D | 7E |  |

Type:	0x00

Command:	0x0D PL:	0x0000

Checksum:	0x0D

4.11.2. Response frame

If get query parameters successfully, a response frame will be returned:

| Header | Type | Command | PL(MSB) | PL(LSB) | Para(MSB) | Para(LSB) | Checksum |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BB | 01 | 0D | 00 | 02 | 10 | 20 | 40 |
| End |  |  |  |  |  |  |  |
| 7E |  |  |  |  |  |  |  |

Type:	0x01

Command:	0x0D PL:	0x0002

Query Parameter:0x1020

Checksum:	0x40

In the above example, the query parameters are as follows:

DR=8, M=1, TRext=Use pilot tone, Sel=00, Session=00, Target=A, Q=4

where:

DR (1 bit):	DR=8(1’b0), DR=64/3(1’b1). Only DR=8 is supported

M (2 bit):	M=1(2’b00), M=2(2’b01), M=4(2’b10), M=8(2’b11). Only M=1 is supported. TRext (1 bit):	No pilot tone(1’b0), Use pilot tone(1’b1). Only use pilot tone (1’b1) is supported Sel (2 bit):	ALL(2’b00/2’b01), ~SL(2’b10), SL(2’b11)

Session (2 bit):	S0(2’b00), S1(2’b01), S2(2’b10), S3(2’b11) Target (1 bit):	A(1’b0), B(1’b1)

Q (4 bit):	4’b0000-4’b1111

4.12. Set Query Parameters

4.12.1. Command frame

Set parameters of Query. The parameters (2 byte length) are defined as follows: DR(1 bit):	DR=8(1’b0), DR=64/3(1’b1). Only DR=8 is supported

M(2 bit):	M=1(2’b00), M=2(2’b01), M=4(2’b10), M=8(2’b11). Only M=1 is supported TRext(1 bit):	No pilot tone(1’b0), Use pilot tone(1’b1). Only use pilot tone(1’b1) is supported Sel(2 bit):	ALL(2’b00/2’b01), ~SL(2’b10), SL(2’b11)

Session(2 bit):	S0(2’b00), S1(2’b01), S2(2’b10), S3(2’b11) Target(1 bit):	A(1’b0), B(1’b1)

Q(4 bit):	4’b0000-4’b1111

Example (DR=8, M=1, TRext=Use pilot tone, Sel=00, Session=00, Target=A, Q=4):

| Header | Type | Command | PL(MSB) | PL(LSB) | Para(MSB) | Para(LSB) | Checksum |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BB | 00 | 0E | 00 | 02 | 10 | 20 | 40 |
| End |  |  |  |  |  |  |  |
| 7E |  |  |  |  |  |  |  |

Type:	0x00

Command:	0x0E PL:	0x0002

Parameter:	0x1020

Checksum:	0xC6

4.12.2. Response frame

If set query parameters successfully, a response frame will be returned:

| Header | Type | Command | PL(MSB) | PL(LSB) | Parameter | Checksum | End |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BB | 01 | 0E | 00 | 01 | 00 | 10 | 7E |

Type:	0x01

Command:	0x0E PL:	0x0001

Parameter:	0x00

Checksum:	0x10

4.13. Set Region

4.13.1. Command frame

Set region for radio frequency regulation. Example (China band):

| Header | Type | Command | PL(MSB) | PL(LSB) | Region | Checksum | End |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BB | 00 | 07 | 00 | 01 | 01 | 09 | 7E |

Type:	0x00

Command:	0x07

PL:	0x0001

Region:	0x01

Checksum:	0x09

Region Code Table：

| Region | Parameter |
| --- | --- |
| China 900MHz | 01 |
| China 800MHz | 04 |
| USA | 02 |
| Europe | 03 |
| Korea | 06 |

4.13.2. Response frame

If region is set successfully, a response frame will be returned:

| Header | Type | Command | PL(MSB) | PL(LSB) | Parameter | Checksum | End |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BB | 01 | 07 | 00 | 01 | 00 | 09 | 7E |

Type:	0x01

Command:	0x07

PL:	0x0001

Parameter:	0x00

Checksum:	0x09

4.14. Get Region

4.14.1. Command frame

Get module current region command.

| Header | Type | Command | PL(MSB) | PL(LSB) | Checksum | End |
| --- | --- | --- | --- | --- | --- | --- |
| BB | 00 | 08 | 00 | 00 | 08 | 7E |

Type:	0x00

Command:	0x08

PL:	0x0000

Checksum:	0x08

4.14.2. Response frame

A response frame includes current working region (in this example, region is China 900MHz) will be returned:

| Header | Type | Command | PL(MSB) | PL(LSB) | Region | Checksum | End |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BB | 01 | 08 | 00 | 01 | 01 | 0B | 7E |

Type:	0x01

Command:	0x08

PL:	0x0001

Region:	0x01

Checksum:	0x0B

4.15. Set Channel

4.15.1. Command frame

Set the current working channel. Example (In China band, set the working channel to 920.125MHz)

| Header | Type | Command | PL(MSB) | PL(LSB) | CH Index | Checksum | End |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BB | 00 | AB | 00 | 01 | 01 | AC | 7E |

Type:	0x00

Command:	0xAB PL:	0x0001

Channel Index:	0x01

Checksum:	0xAC

Calculate channel index from channel central frequency (Freq_CH is the channel central frequency): China 900MHz band channel index:

CH_Index = (Freq_CH-920.125M)/0.25M

China 800MHz band channel index: CH_Index = (Freq_CH-840.125M)/0.25M

Amercian band channel index: CH_Index = (Freq_CH-902.25M)/0.5M

European band channel index: CH_Index = (Freq_CH-865.1M)/0.2M

Korean band channel index: CH_Index = (Freq_CH-917.1M)/0.2M

4.15.2. Response frame

If set channel successfully, a response frame will be returned:

| Header | Type | Command | PL(MSB) | PL(LSB) | Parameter | Checksum | End |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BB | 01 | AB | 00 | 01 | 00 | AD | 7E |

Type:	0x01

Command:	0xAB

长度 PL:	0x0001

Parameter:	0x00

Checksum:	0xAD

4.16. Get Channel

4.16.1. Command frame

Get the current working channel:

| Header | Type | Command | PL(MSB) | PL(LSB) | Checksum | End |
| --- | --- | --- | --- | --- | --- | --- |
| BB | 00 | AA | 00 | 00 | AA | 7E |

Type:	0x00

Command:	0xAA PL:	0x0000

Checksum:	0xAA

4.16.2. Response frame

If get channel successfully, a response frame will be returned:

| Header | Type | Command | PL(MSB) | PL(LSB) | Parameter | Checksum | End |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BB | 01 | AA | 00 | 01 | 00 | AC | 7E |

Type:	0x01

Command:	0xAA PL:	0x0001

Parameter:	0x00(Channel_Index: 0x00) Checksum:	0xAC

Calculate channel frequency from channel index: China 900MHz band:

Freq_CH = CH_Index * 0.25M + 920.125M

China 800MHz band:

Freq_CH = CH_Index * 0.25M + 840.125M

American band:

Freq_CH = CH_Index * 0.5M + 902.25M

European band:

Freq_CH = CH_Index * 0.2M + 865.1M

Korean band:

Freq_CH = CH_Index * 0.2M + 917.1M

4.17. Frequency Hopping

4.17.1. Command frame

Enable/Disable frequency hopping:

| Header | Type | Command | PL(MSB) | PL(LSB) | Parameter | Checksum | End |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BB | 00 | AD | 00 | 01 | FF | AD | 7E |

Type:	0x00

Command:	0xAD PL:	0x0001

Parameter:	0xFF(0xFF: enable frequency hopping; 0x00: disable frequency hopping) Checksum:	0xAD

4.17.2. Response frame

If frequency hopping is set successfully, a response frame will be returned:

| Header | Type | Command | PL(MSB) | PL(LSB) | Parameter | Checksum | End |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BB | 01 | AD | 00 | 01 | 00 | AF | 7E |

Type:	0x01

Command:	0xAD PL:	0x0001

Parameter:	0x00

Checksum:	0xAF

4.18. Get TX Power

4.18.1. Command frame

Get the current transmitting power:

| Header | Type | Command | PL(MSB) | PL(LSB) | Checksum | End |
| --- | --- | --- | --- | --- | --- | --- |
| BB | 00 | B7 | 00 | 00 | B7 | 7E |

Type:	0x00

Command:	0xB7

PL:	0x0000

Checksum:	0xB7

4.18.2. Response frame

If get TX power successfully, a response frame will be returned:

| Header | Type | Command | PL(MSB) | PL(LSB) | Pow(MSB) | Pow(LSB) | Checksum |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BB | 01 | B7 | 00 | 02 | 07 | D0 | 91 |
| End |  |  |  |  |  |  |  |
| 7E |  |  |  |  |  |  |  |

Type:	0x01

Command:	0xB7

PL:	0x0002

Power:	0x07D0(in the unit of dBm. e.g. 0x07D0_Hex = 2000_Dec: 20dBm) Checksum:	0x91

4.19. Set TX Power

4.19.1. Command frame

Set current transmitting power:

| Header | Type | Command | PL(MSB) | PL(LSB) | Pow(MSB) | Pow(LSB) | Checksum |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BB | 00 | B6 | 00 | 02 | 07 | D0 | 8F |
| End |  |  |  |  |  |  |  |
| 7E |  |  |  |  |  |  |  |

Type:	0x00

Command:	0xB6

PL:	0x0002

Pow:	0x07D0(in the unit of dBm) Checksum:	0x8F

4.19.2. Response frame

The response frame is as follow:

| Header | Type | Command | PL(MSB) | PL(LSB) | Parameter | Checksum | End |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BB | 01 | B6 | 00 | 01 | 00 | B8 | 7E |

Type:	0x01

Command:	0xB6

PL:	0x0001

Parameter:	0x00

Checksum:	0xB8

4.20. Continuous Wave

4.20.1. Command frame

On/Off continuous wave output:

| Header | Type | Command | PL(MSB) | PL(LSB) | Parameter | Checksum | End |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BB | 00 | B0 | 00 | 01 | FF | B0 | 7E |

Type:	0x00

Command:	0xB0

PL:	0x0001

Parameter:	0xFF(0xFF: on; 0x00: off) Checksum:	0xB0

4.20.2. Response frame

If set continuous wave successfully, a response frame will be returned:

| Header | Type | Command | PL(MSB) | PL(LSB) | Parameter | Checksum | End |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BB | 01 | B0 | 00 | 01 | 00 | B2 | 7E |

Type:	0x01

Command:	0xB0

PL:	0x0001

Parameter:	0x00

Checksum:	0xB2

4.21. Get Modem Parameters

4.21.1. Command frame

To get the current key modem parameters that includes: the mixer gain, the intermediate frequency (baseband) amplifier gain and the threshold for demodulation (the tag returned signal with RSSI higher than threshold will be demodulated):

| Header | Type | Command | PL(MSB) | PL(LSB) | Checksum | End |
| --- | --- | --- | --- | --- | --- | --- |
| BB | 00 | F1 | 00 | 00 | F1 | 7E |

Type:	0x00

Command:	0xF1

PL:	0x0000

Checksum:	0xF1

4.21.2. Response frame

If get modem parameters successfully, a response frame will be returned:

| Header | Type | Command | PL(MSB) | PL(LSB) | Mixer_G | IF_G | Thrd(MSB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BB | 01 | F1 | 00 | 04 | 03 | 06 | 01 |
| Thrd(LSB) | Checksum | End |  |  |  |  |  |
| B0 | B0 | 7E |  |  |  |  |  |

Type:	0x01

Command:	0xF1

PL:	0x0004

Mixer_G:	0x03(Mixer gain: 9dB) IF_G:	0x06(IF AMP gain: 36dB)

Threshold:	0x01B0 (The smaller the threshold is, the weaker tag returned signal could be received, but with the higher package error rate. So small threshold is for short communication, like inventory, to get longer reading distance. And large threshold is recommended for long communication, like read/write, to increase the success rate. 0x01B0 is a typical small threshold)

Checksum:	0xB0

Mixer Gain

| Type | Mixer_G(dB) |
| --- | --- |
| 0x00 | 0 |
| 0x01 | 3 |
| 0x02 | 6 |
| 0x03 | 9 |
| 0x04 | 12 |
| 0x05 | 15 |
| 0x06 | 16 |

IF AMP Gain

| Type | IF_G(dB) |
| --- | --- |
| 0x00 | 12 |
| 0x01 | 18 |
| 0x02 | 21 |
| 0x03 | 24 |

|  | 0x04 | 27 |  |
| --- | --- | --- | --- |
|  | 0x05 | 30 |  |
|  | 0x06 | 36 |  |
|  | 0x07 | 40 |  |

4.22. Set Modem Parameters

4.22.1. Command frame

T o set the key modem parameters which includes: the mixer gain, the IF amplifier gain and the threshold for demodulation:

| Header | Type | Command | PL(MSB) | PL(LSB) | Mixer_G | IF_G | Thrd(MSB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BB | 00 | F0 | 00 | 04 | 03 | 06 | 01 |
| Thrd(LSB) | Checksum | End |  |  |  |  |  |
| B0 | AE | 7E |  |  |  |  |  |

Type:	0x00

Command:	0xF0

PL:	0x0004

Mixer_G:	0x03(Mixer gain: 9dB) IF_G:	0x06(IF AMP gain: 36dB)

Threshold:	0x01B0(The smaller the threshold is, the weaker tag returned signal could be received, but with the higher package error rate. So small threshold is for short communication, like inventory, to get longer reading distance. And large threshold is recommended for long communication, like read/write, to increase the success rate. 0x01B0 is a typical small threshold)

Checksum:	0xAE

Mixer Gain

| Type | Mixer_G(dB) |
| --- | --- |
| 0x00 | 0 |
| 0x01 | 3 |
| 0x02 | 6 |
| 0x03 | 9 |
| 0x04 | 12 |
| 0x05 | 15 |
| 0x06 | 16 |

IF AMP Gain

| Type | IF_G(dB) |
| --- | --- |
| 0x00 | 12 |
| 0x01 | 18 |
| 0x02 | 21 |
| 0x03 | 24 |
| 0x04 | 27 |
| 0x05 | 30 |
| 0x06 | 36 |
| 0x07 | 40 |

4.22.2. Response frame

If set modem parameters successfully, a response frame will be returned:

| Header | Type | Command | PL(MSB) | PL(LSB) | Parameter | Checksum | End |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BB | 01 | F0 | 00 | 01 | 00 | F2 | 7E |

Type:	0x01

Command:	0xF0

PL:	0x0001

Parameter:	0x00

Checksum:	0xF1

4.23. Scan Jammer

4.23.1. Command frame

To scan the jammer signal strength at the RF input port of the reader chip (a continuous wave will be transmitted at the current set power when doing the scan). The channel to be scan is determined by the current frequency band setting.

| Header | Type | Command | PL(MSB) | PL(LSB) | Checksum | End |
| --- | --- | --- | --- | --- | --- | --- |
| BB | 00 | F2 | 00 | 00 | F2 | 7E |

Type:	0x00

Command:	0xF2

PL:	0x0000

Checksum:	0xF2

4.23.2. Response frame

The scan jammer result will be returned by a response frame. E.g. in China 900MHz band, there are 20 channels. If the scan jammer is executed successfully, a response frame will be returned:

| Header | Type | Command | PL(MSB) | PL(LSB) | CH_L | CH_H | JMR(MSB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BB | 01 | F2 | 00 | 16 | 00 | 13 | F2 |
| F1 | F0 | EF | EC | EA | E8 | EA | EC |
| EE | F0 | F1 | F5 | F5 | F5 | F6 | F5 |
|  |  | JMR(LSB) | Checksum | End |  |  |  |
| F5 | F5 | F5 | DD | 7E |  |  |  |

Type:	0x01

Command:	0xF2

PL:	0x0016

CH_L:	0x00 (the first channel index is: 0) CH_H:	0x13 (the last channel index is: 19)

JMR:	0xF2F1F0EFECEAE8EAECEEF0F1F5F5F5F6F5F5F5F5 (0xF2: -14dBm) Checksum:	0xDD

4.24. Scan Channel (LBT)

4.24.1. Command frame

Scan the channel power at the RF input power of the reader chip:

| Header | Type | Command | PL(MSB) | PL(LSB) | Checksum | End |
| --- | --- | --- | --- | --- | --- | --- |
| BB | 00 | F3 | 00 | 00 | F3 | 7E |

Type:	0x00

Command:	0xF3

PL:	0x0000

Checksum:	0xF3

4.24.2. Response frame

The scan channel result will be returned by a response frame. E.g. in China 900MHz band, there are 20 channels. If the scan channel is executed successfully, a response frame will be returned:

| Header | Type | Command | PL(MSB) | PL(LSB) | CH_L | CH_H | RSSI(MSB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BB | 01 | F3 | 00 | 16 | 00 | 13 | BA |
| BA | BA | BA | BA | BA | BA | BA | BA |
| BA | BA | BA | BA | BA | BA | BA | BA |
|  |  | RSSI(LSB) | Checksum | End |  |  |  |
| BA | BA | BA | A5 | 7E |  |  |  |

Type:	0x01

Command:	0xF2

PL:	0x0016

CH_L:	0x00(the first channel index is: 0) CH_H:	0x13(the last channel index is: 19)

JMR:	0xBABABABABABABABABABABABABABABABABABABABA(0xBA: -70dBm, is the lower limit of the RSSI)

Checksum:	0xDD

4.25. GPIO

4.25.1. Command frame

Set the IO direction, set/get IO status:

| Header | Type | Command | PL(MSB) | PL(LSB) | Parameter0 | Parameter1 | Parameter2 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BB | 01 | 1A | 00 | 03 | 00 | 04 | 01 |
| Checksum | End |  |  |  |  |  |  |
| 22 | 7E |  |  |  |  |  |  |

Type:	0x00

Command:	0x1A PL:	0x0003

Parameter:	0x00 0x04 0x01

Checksum:	0x22

Parameters:

| Index | Description | Length | Operation |
| --- | --- | --- | --- |
| 0 | Parameter0 | 1 byte | Operation: 0x00：Set IO direction; 0x01：Set IO status； 0x02：Get IO status; Current IO is defined by parameter1; |
| 1 | Parameter1 | 1 byte | Range: 0x01-0x04: IO1-IO4 |
| 2 | Parameter2 | 1 byte | Range: 0x00-0x01。 When Parameter0=0x02, Parameter2 will be ignored. |

| Parameter0 | Parameter2 | Description |
| --- | --- | --- |
| 0x00 | 0x00 | IO direction: Input |
| 0x00 | 0x01 | IO direction: Output |
| 0x01 | 0x00 | IO status: Low |
| 0x01 | 0x01 | IO status: High |

4.25.2. Response frame

If the IO is set successfully, a response frame will be returned:

| Header | Type | Command | PL(MSB) | PL(LSB) | Parameter0 | Parameter1 | Parameter2 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BB | 01 | 1A | 00 | 03 | 00 | 04 | 01 |
| Checksum | End |  |  |  |  |  |  |
| 23 | 7E |  |  |  |  |  |  |

Type:	0x01

Command:	0x1A PL:	0x0003

Parameter:	0x00 0x04 0x01

Checksum:	0x23

Index	Description	Length	说明

0	Parameter0	1 byte	Operation:

0x00：Set IO direction;

0x01：Set IO status；

0x02：Get IO status;

Current IO is defined by parameter1;

1	Parameter 1	1 byte	Range: 0x01-0x04: IO1-IO4

2	Parameter 2	1 byte	Range: 0x00-0x01。

Parameter0	Parameter2	Description

0x00	0x00	Set IO direction fail

0x00	0x01	Set IO direction success

0x01	0x00	Set IO status fail

0x01	0x01	Set IO status success

0x02	0x00	IO status is low

0x02	0x01	IO status is high

4.26. Module Sleep

4.26.1. Command frame

The Module Sleep command can make the module stay a low power status. Any byte sent by UART can wake up the module and the byte will be discarded. This command will also reset the M100/QM100 chip. The firmware will be downloaded to M100/QM100 chip after waking up. So some parameters might be reset.

| Header | Type | Command | PL(MSB) | PL(LSB) | Checksum | End |
| --- | --- | --- | --- | --- | --- | --- |
| BB | 00 | 17 | 00 | 00 | 17 | 7E |

Type:	0x00

Command:	0x17

PL:	0x0000

Checksum:	0x17

4.26.2. Response frame

The response frame is defined as blow：

| Header | Type | Command | PL(MSB) | PL(LSB) | Parameter | Checksum | End |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BB | 01 | 17 | 00 | 01 | 00 | 19 | 7E |

Type:	0x01

Command:	0x17

PL:	0x0001

Parameter:	0x00

Checksum:	0x19

4.27. Set Module Idle Time

4.27.1. Command frame

This command can set how many minutes the module stays idle before it enters into sleep mode automatically.

| Header | Type | Command | PL(MSB) | PL(LSB) | Parameter | Checksum | End |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BB | 00 | 1D | 00 | 01 | 02 | 20 | 7E |

Type:	0x00

Command:	0x1D PL:	0x0001

Parameter:	0x02(sleep after 2 minutes no operation，range 1~9minutes，0x00 means no

automatic sleep)

Checksum:	0x20

4.27.2. Response frame

The response frame is defined as blow：

| Header | Type | Command | PL(MSB) | PL(LSB) | Parameter | Checksum | End |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BB | 01 | 1D | 00 | 01 | 02 | 21 | 7E |

Type:	0x01

Command:	0x1D PL:	0x0001

Parameter:	0x02（Idle time）

Checksum:	0x21

4.28. NXP ReadProtect/Reset ReadProtect

NXP ReadProtect/Reset ReadProtect

NXP G2X tags support ReadProtect/Reset ReadProtect commands. The tag’s ProtectEPC and ProtectTID bits will be set to ‘1’ after executing ReadProtect successfully, and the tag will be in data protection status. A Reset ReadProtect command should be executed if a user wants the tag return to normal status. Before this command, a set select parameter command should be sent first to select a signal tag.

4.28.1. Command frame

ReadProtect/Reset ReadProtect command is defined as blow:

| Header | Type | Command | PL(MSB) | PL(LSB) | AP(MSB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BB | 00 | E1 | 00 | 05 | 00 | 00 | FF |
| AP(LSB) | Reset | Checksum | End |  |  |  |  |
| FF | 00 | E4 | 7E |  |  |  |  |

Type:	0x00

Command:	0xE1

PL:	0x0005

Access Password:	0x0000FFFF

ReadProtect/Reset ReadProtect:	0x00(0x00: ReadProtect，0x01: Reset ReadProtect) Checksum:	0x0B

4.28.2. Response frame

If a ReadProtect executing successfully，the response frame is：

| Header | Type | Command | PL(MSB) | PL(LSB) | UL | PC(MSB) | PC(LSB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BB | 01 | E1 | 00 | 10 | 0E | 30 | 00 |
| EPC(MSB) |  |  |  |  |  |  |  |
| 30 | 75 | 1F | EB | 70 | 5C | 59 | 04 |
|  |  |  | EPC(LSB) | Parameter | Checksum | End |  |
| E3 | D5 | 0D | 70 | 00 | 3D | 7E |  |

Type:	0x01

Command:	0xE1

PL:	0x0010

PC+EPC length UL:	0x0E PC:	0x3000

EPC:	0x30751FEB705C5904E3D50D70

Parameter:	0x00(executing success) Checksum:	0x3D

If a Reset ReadProtect executing successfully，the response frame is：

| Header | Type | Command | PL(MSB) | PL(LSB) | UL | PC(MSB) | PC(LSB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BB | 01 | E2 | 00 | 10 | 0E | 30 | 00 |
| EPC(MSB) |  |  |  |  |  |  |  |
| 30 | 75 | 1F | EB | 70 | 5C | 59 | 04 |
|  |  |  | EPC(LSB) | Parameter | Checksum | End |  |
| E3 | D5 | 0D | 70 | 00 | 3E | 7E |  |

Type:	0x01

Command:	0xE2

PL:	0x0010

PC+EPC lengthUL:	0x0E PC:	0x3000

EPC:	0x30751FEB705C5904E3D50D70

Parameter:	0x00(executing success) Checksum:	0x3E

If there is no tag response after ReadProtect (Set/Reset parameter is 0x00) command, an error code of

0x2A will be returned:

| Header | Type | Command | PL(MSB) | PL(LSB) | Parameter | Checksum | End |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BB | 01 | FF | 00 | 01 | 2A | 2B | 7E |

Type:	0x01

Command:	0xFF PL:	0x0001

Parameter:	0x2A Checksum:	0x2B

If there is no tag response after Reset ReadProtect (Set/Reset parameter is 0x01) command, an error code of 0x2B will be returned:

| Header | Type | Command | PL(MSB) | PL(LSB) | Parameter | Checksum | End |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BB | 01 | FF | 00 | 01 | 2B | 2C | 7E |

Type:	0x01

Command:	0xFF PL:	0x0001

Parameter:	0x2B Checksum:	0x2C

If the access password is not correct, an error code of 0x16 will be returned, and the tag’s PC+EPC will be sent back:

| Header | Type | Command | PL(MSB) | PL(LSB) | Error Code | UL | PC(MSB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BB | 01 | FF | 00 | 10 | 16 | 0E | 34 |
| PC(LSB) | EPC(MSB) |  |  |  |  |  |  |
| 00 | 30 | 75 | 1F | EB | 70 | 5C | 59 |
|  |  |  |  | EPC(LSB) | Checksum | End |  |
| 04 | E3 | D5 | 0D | 70 | 75 | 7E |  |

Type:	0x01

Command:	0xFF PL:	0x0010

Error Code:	0x16

PC+EPC length:	0x0E PC:	0x3400

EPC:	0x30751FEB705C5904E3D50D70

Checksum:	0x75

4.29. NXP Change EAS

NXP G2X tags support Change EAS command. The tag’s PSF bit will be set to ‘1’ or ‘0’ after executing Change EAS command. The tag will answer EAS_Alarm command when PSF bit is ‘1’, otherwise the tag will not answer EAS_Alarm command when PSF bit is ‘0’. Before this command, a set select parameter command should be sent first to select a signal tag.

4.29.1. Command frame

Change EAS command is defined as blow:

| Header | Type | Command | PL(MSB) | PL(LSB) | AP(MSB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BB | 00 | E3 | 00 | 05 | 00 | 00 | FF |
| AP(LSB) | PSF | Checksum | End |  |  |  |  |
| FF | 01 | E7 | 7E |  |  |  |  |

Type:	0x00

Command:	0xE3

PL:	0x0005

Access Password:	0x0000FFFF

Set/Reset:	0x01(0x01 means set PSF bit to ’1’, 0x00 means set PSF to ’0’) Checksum:	0xE7

4.29.2. Response frame

If Change EAS command executing successfully，the response frame is:

| Header | Type | Command | PL(MSB) | PL(LSB) | UL | PC(MSB) | PC(LSB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BB | 01 | E3 | 00 | 10 | 0E | 30 | 00 |
| EPC(MSB) |  |  |  |  |  |  |  |
| 30 | 75 | 1F | EB | 70 | 5C | 59 | 04 |
|  |  |  | EPC(LSB) | Parameter | Checksum | End |  |
| E3 | D5 | 0D | 70 | 00 | 3F | 7E |  |

Type:	0x01

Command:	0xE3

PL:	0x0010

PC+EPC length UL:	0x0E PC:	0x3000

EPC:	0x30751FEB705C5904E3D50D70

Parameter:	0x00(success) Checksum:	0x3F

If there is no tag response after Change EAS command, an error code of 0x1B will be returned:

| Header | Type | Command | PL(MSB) | PL(LSB) | Parameter | Checksum | End |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BB | 01 | FF | 00 | 01 | 1B | 1C | 7E |

Type:	0x01

Command:	0xFF PL:	0x0001

Parameter:	0x1B Checksum:	0x1C

If the access password is not correct, an error code of 0x16 will be returned, and the tag’s PC+EPC will be sent back:

| Header | Type | Command | PL(MSB) | PL(LSB) | Error Code | UL | PC(MSB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BB | 01 | FF | 00 | 10 | 16 | 0E | 34 |
| PC(LSB) | EPC(MSB) |  |  |  |  |  |  |
| 00 | 30 | 75 | 1F | EB | 70 | 5C | 59 |
|  |  |  |  | EPC(LSB) | Checksum | End |  |
| 04 | E3 | D5 | 0D | 70 | 75 | 7E |  |

Type:	0x01

Command:	0xFF PL:	0x0010

Error Code:	0x16

PC+EPC length:	0x0E PC:	0x3400

EPC:	0x30751FEB705C5904E3D50D70

Checksum:	0x75

4.30. NXP EAS_Alarm

NXP G2X tags support EAS_Alarm command. The tag will answer 64bits EAS-Alarm code immediately after

EAS-alarm command. The tag will answer EAS-Alarm command only when PSF bit is set to ‘1’.

4.30.1. Command frame

EAS_Alarm command is defined as blow:

| Header | Type | Command | PL(MSB) | PL(LSB) | Checksum | End |
| --- | --- | --- | --- | --- | --- | --- |
| BB | 00 | E4 | 00 | 00 | E4 | 7E |

Type:	0x00

Command:	0xE4

PL:	0x0000

Checksum:	0xE4

4.30.2. Response frame

If EAS_Alarm command executing successfully and the tag answers the right 64bits EAS-Alarm code, the response frame is:

| Header | Type | Command | PL(MSB) | PL(LSB) | EAS-Alarm code(MSB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BB | 01 | E4 | 00 | 08 | 69 | 0A | EC |
|  |  |  |  | EAS-Alarm code(LSB) | Checksum | End |  |
| 7C | D2 | 15 | D8 | F9 | 80 | 7E |  |

Type:	0x01

Command:	0xE3

PL:	0x0001

Parameter:	0x00

Checksum:	0xE5

If there is no tag response after Change EAS command, an error code of 0x1D will be returned:

| Header | Type | Command | PL(MSB) | PL(LSB) | Parameter | Checksum | End |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BB | 01 | FF | 00 | 01 | 1D | 1E | 7E |

Type:	0x01

Command:	0xFF PL:	0x0001

Parameter:	0x1D Checksum:	0x1E

4.31. NXP ChangeConfig

Some series of NXP G2X tags such as G2iM and G2iM+ support ChangeConfig command. This command can read or modify the 16bits Config-Word. The Config-Word is stored at address 20h of Bank 01 (EPC MemBank). It can be read or write by standard READ/WRITE command. The bits to be toggled in the configuration register need to be set to '1'. Before this command, a set select parameter command should be sent first to select a signal tag.

4.31.1. Command frame

ChangeConfig command is defined as blow:

| Header | Type | Command | PL(MSB) | PL(LSB) | AP(MSB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BB | 00 | E0 | 00 | 06 | 00 | 00 | FF |
| AP(LSB) | Config(MSB) | Config (LSB) | Checksum | End |  |  |  |
| FF | 00 | 00 | E4 | 7E |  |  |  |

Type:	0x00

Command:	0xE0

PL:	0x0006

Access Password:	0x0000FFFF

Config-Word:	0x0000(all 0 means response is unchanged Config-Word, same as read) Checksum:	0xE4

4.31.2. Response frame

If ChangeConfig executing successfully, the response is:

| Header | Type | Command | PL(MSB) | PL(LSB) | UL | PC(MSB) | PC(LSB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BB | 01 | E0 | 00 | 11 | 0E | 30 | 00 |
| EPC(MSB) |  |  |  |  |  |  |  |
| 30 | 75 | 1F | EB | 70 | 5C | 59 | 04 |
|  |  |  | EPC(LSB) | Config(MSB) | Config(LSB) | Checksum | End |
| E3 | D5 | 0D | 70 | 00 | 41 | 7E | 7E |

Type:	0x01

Command:	0xE0

PL:	0x0011

UL:	0x0E PC:	0x3000

EPC:	0x30751FEB705C5904E3D50D70

Config-Word:	0x0041

Checksum:	0x7E

If there is no tag response after ChangeConfig command, an error code of 0x1A will be returned:

| Header | Type | Command | PL(MSB) | PL(LSB) | Parameter | Checksum | End |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BB | 01 | FF | 00 | 01 | 1A | 1B | 7E |

Type:	0x01

Command:	0xFF PL:	0x0001

Parameter:	0x1A Checksum:	0x1B

If the access password is not correct, an error code of 0x16 will be returned, and the tag’s PC+EPC will be

sent back:

| Header | Type | Command | PL(MSB) | PL(LSB) | Error Code | UL | PC(MSB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BB | 01 | FF | 00 | 10 | 16 | 0E | 34 |
| PC(LSB) | EPC(MSB) |  |  |  |  |  |  |
| 00 | 30 | 75 | 1F | EB | 70 | 5C | 59 |
|  |  |  |  | EPC(LSB) | Checksum | End |  |
| 04 | E3 | D5 | 0D | 70 | 75 | 7E |  |

Type:	0x01

Command:	0xFF PL:	0x0010

Error Code:	0x16

PC+EPC length:	0x0E PC:	0x3400

EPC:	0x30751FEB705C5904E3D50D70

Checksum:	0x75

4.32. Impinj Monza QT

Impinj Monza 4QT tags support QT command that can modify the QT Control Word. Setting the QT_SR bit can reduce the tag’s operation range and modifying the QT_MEM bit can switch the tag to use Public Memory Map or Private Memory Map. Before this command, a set select parameter command should be sent first to select a signal tag.

4.32.1. Command frame

QT command is defined as blow (set QT_MEM to 1 and write it to Non-volatilizing memory in this example):

| Header | Type | Command | PL(MSB) | PL(LSB) | AP(MSB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BB | 00 | E5 | 00 | 08 | 00 | 00 | FF |
| AP(LSB) | Read/Write | Persistence | Payload0 | Payload1 | Checksum | End |  |
| FF | 01 | 01 | 40 | 00 | 2D | 7E |  |

Type:	0x00

Command:	0xE5

PL:	0x0008

Access Password:	0x0000FFFF

Read/Write:	0x01(0x00: Read，0x01: Write)

Persistence:	0x01(0x00: write to volatilizing memory, 0x01: write it to Non-volatilizing memory, should always be 0x01)

Payload:	0x4000(QT Control，two MSB bits is QT_SR and QT_MEM)

Checksum:	0x2D

4.32.2. Response frame

If QT command executing successfully and Read/Write field is 0x00, the response frame is:

| Header | Type | Command | PL(MSB) | PL(LSB) | UL | PC(MSB) | PC(LSB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BB | 01 | E5 | 00 | 11 | 0E | 30 | 00 |
| EPC(MSB) |  |  |  |  |  |  |  |
| 30 | 75 | 1F | EB | 70 | 5C | 59 | 04 |
|  |  |  | EPC(LSB) | QT Control0 | QT Control1 | Checksum | End |
| E3 | D5 | 0D | 70 | 00 | 00 | 42 | 7E |

Type:	0x01

Command:	0xE5

PL:	0x0011

PC+EPC length UL:	0x0E PC:	0x3000

EPC:	0x30751FEB705C5904E3D50D70

QT Control Word:	0x0000

Checksum:	0x42

If QT command executing successfully and Read/Write field is 0x01, the response frame is:

| Header | Type | Command | PL(MSB) | PL(LSB) | UL | PC(MSB) | PC(LSB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BB | 01 | E6 | 00 | 10 | 0E | 30 | 00 |
| EPC(MSB) |  |  |  |  |  |  |  |
| 30 | 75 | 1F | EB | 70 | 5C | 59 | 04 |
|  |  |  | EPC(LSB) | Parameter | Checksum | End |  |

|  | E3 | D5 | 0D | 70 | 00 | 42 | 7E |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Type:	0x01

Command:	0xE6

PL:	0x0010

PC+EPC length UL:	0x0E PC:	0x3000

EPC:	0x30751FEB705C5904E3D50D70

Parameter:	0x00(success) Checksum:	0x42

If there is no tag response after QT command, an error code of 0x2E will be returned:

| Header | Type | Command | PL(MSB) | PL(LSB) | Parameter | Checksum | End |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BB | 01 | FF | 00 | 01 | 2E | 2F | 7E |

Type:	0x01

Command:	0xFF PL:	0x0001

Parameter:	0x2E Checksum:	0x2F

If the access password is not correct, an error code of 0x16 will be returned, and the tag’s PC+EPC will be sent back:

| Header | Type | Command | PL(MSB) | PL(LSB) | Error Code | UL | PC(MSB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BB | 01 | FF | 00 | 10 | 16 | 0E | 34 |
| PC(LSB) | EPC(MSB) |  |  |  |  |  |  |
| 00 | 30 | 75 | 1F | EB | 70 | 5C | 59 |
|  |  |  |  | EPC(LSB) | Checksum | End |  |
| 04 | E3 | D5 | 0D | 70 | 75 | 7E |  |

Type:	0x01

Command:	0xFF PL:	0x0010

Error Code:	0x16

PC+EPC length:	0x0E PC:	0x3400

EPC:	0x30751FEB705C5904E3D50D70

Checksum:	0x75

5.	Summary

| Code | Description |
| --- | --- |
| 0x03 | Get Module Information |
| 0x22 | Single Inventory |
| 0x27 | Multiple Inventory |
| 0x28 | Stop Multiple Inventory |
| 0x0C | Set Select Parameters |
| 0x12 | Set Select Mode |
| 0x39 | Read |
| 0x49 | Write |
| 0x82 | Lock |
| 0x65 | Kill |
| 0x0D | Get Query Parameters |
| 0x0E | Set Query Parameters |
| 0x07 | Set Region |
| 0x08 | Get Region |
| 0xAB | Set Channel |
| 0xAA | Get Channel |
| 0xAD | Frequency Hopping |
| 0xB7 | Get TX Power |
| 0xB6 | Set TX Power |
| 0xB0 | Continuous Wave |
| 0xF1 | Get Modem Parameters |
| 0xF0 | Set Modem Parameters |
| 0xF2 | Scan Jammer |
| 0xF3 | Scan Channel |
| 0x1A | GPIO |
| 0x17 | Module Sleep |
| 0x1D | Set Module Idle Time |
| 0xE1 | NXP ReadProtec/Reset ReadProtect |
| 0xE3 | NXP Change EAS |
| 0xE4 | NXP EAS-Alarm |
| 0xE0 | NXP ChangeConfig |
| 0xE5 | Impinj Monza 4 QT |

6.   Error Code

If a command execution is fail, a fail response frame will be returned which includes an error code to indicate the error type. E.g. if inventory command is fail by no tag response or CRC error, an error code "0x15" will be returned in a response frame:

| Header | Type | Command | PL(MSB) | PL(LSB) | Parameter | Checksum | End |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BB | 01 | FF | 00 | 01 | 15 | 16 | 7E |

Type:	0x01

Command:	0xFF (0xFF indicates command fail) PL:	0x01

Parameter:	0x15 (error code) Checksum:	0x16

Error code summary:

| Type | Code | Description |
| --- | --- | --- |
| Command Error | 0x17 | Command code error |
| FHSS Fail | 0x20 | Frequency hopping time out. All channel are occupied. |
| Inventory Fail | 0x15 | Inventory fail. No tag response or CRC error. |
| Access Fail | 0x16 | Access fail. May caused by password error. |
| Read Fail | 0x09 | Read fail. No tag response or CRC error. |
| Read Error | 0xA0 ¦ Error code | Read error. Error code (0xA0 ¦ Error code) indication refer to Tag-error code in the next table. |
| Write Fail | 0x10 | Write fail. No tag response or CRC error. |
| Write Error | 0xB0 ¦ Error code | Write error. Error code (0xB0 ¦ Error code) indication refer to Tag-error code in the next table. |
| Lock Fail | 0x13 | Lock fail. No tag response or CRC error. |
| Lock Error | 0xC0 ¦ Error code | Lock error. Error code (0xC0 ¦ Error code) indication refer to Tag-error code in the next table. |
| Kill Fail | 0x13 | Kill fail. No tag response or CRC error. |
| Kill Error | 0xD0 ¦ Error code | Kill error. Error code (0xC0 ¦ Error code) indication refer to Tag-error code in the next table. |

EPC Gen2 protocol specified error code:

Tag error-code

| Error-code Support | Error Code | Error code Name | Error Description |
| --- | --- | --- | --- |
| Error-specific | 000000002 | Other error | Other errors not described in this table. |
|  | 000000112 | Memory overrun | The given memory area does not exist. Or the tag does not support the given EPC length, e.g. XPC. |
|  | 000001002 | Memory locked | The given memory area is locked and/or permanent locked. And the lock status is not readable or not writeable. |
|  | 000010112 | Insufficient power | Tag has no enough power to write. |
| Non-specific | 000011112 | Non-specific error | Tag does not support error-code. |

NXP G2X tags’ specific error code：

| ChangeConfig Fail | 0x1A | ChangeConfig failed. No tag response or CRC error. |
| --- | --- | --- |
| ReadProtect Fail | 0x2A | ReadProtect failed. No tag response or CRC error. |
| Reset ReadProtect Fail | 0x2B | Reset ReadProtect failed. No tag response or CRC error. |
| Change EAS Fail | 0x1B | Change EAS failed. No tag response or CRC error. |
| EAS_Alarm Fail | 0x1D | EAS_Alarm failed. No correct Alarm Code response. |
| Specific commands error code | 0xE0 ¦ Error code | Specific commands error code. Error code (0xE0 ¦ Error code) indication refer to Tag-error code in the EPC Gen2 protocol. |

Impinj Monza QT tags’ specific error code:

| QT Fail | 0x2E | QT failed. No tag response or CRC error. |
| --- | --- | --- |
| Specific commands error code | 0xE0 ¦ Error code | Specific commands error code. Error code (0xE0 ¦ Error code) indication refer to Tag-error code in the EPC Gen2 protocol. |